FlySky FS-i6S Firmware Upgrade Steps:

Version Updating:

flysky i6s 1.10 22-Mar-2016 to flysky i6s 1.11 30-Jun

- Need to bind with receiver again, 2. The menu set contents will be reset, please reset your transmitter.

flysky i6s 1.11 07-Jun-2016 to flysky i6s 1.11 30-Jun

- Need to reset and calibrate the stick date. Please follow the calibration process to calibrate your transmitter.

Upgrade Process:

- Connected the transmitter to the computer via USB cable.

- Enter the Setup menu in the transmitter and navigate the Firmware Update section.

- Open the firmware upgrade tool in your PC.

- In the Firmware Update Section of the transmitter press Continue and the transmitter will enter the update mode.

- Once the transmitter enters the update mode, you will see it listed in the software. Press Update in the software and the update process will begin. It should take a few seconds for the update to take place and then the transmitter will reboot.

- Close the software and unplug the transmitter.

- In the Setup section of the transmitter navigate to the About FS-i6S section to confirm the correct firmware.

Calibration Process:

Once the transmitter’s software has been successfully updated, it must be calibrated. Follow the following steps to calibrate your transmitter:

- Enter the Setup menu and navigate to the Stick Adjust section to initiate the calibration process.

- Simultaneously move both sticks up, down, left, right and then place the sticks at 50% or centered.

- Move the two dials on the transmitter all the way left and right

- Navigate to the Channels Section of the transmitter and verify that all the channels work accordingly.